Ore Reserve Methodology

MM Pty Ltd

Location Plan

The Geological Resource

The Geological Resource at The NVG Project consists of three zones, the Main Vein, The Hanging Wall Split and The Footwall Splay.    All three zones were modelled in this exercise.    The modeling method was as follows :

Validation of basic data, Drillhole locations, Drillhole Survey Information, Downhole information.

Validation of the modeling constraints.    The modeling constraints were defined by outlines digitized on section.    These sectional outlines were then reinterpreted into plan view.

The updated sectional outlines were then used to update the plan view interpretation

The assay data within the Vein was then composited down the hole to an interval of 0.5 m.    The close compositing was used to ensure that grade modeled was not diluted by any grade values from within the country rock.

The composite process resulted in some composites adjacent to the footwall, which did not have a length of 0.5 m.    These were resolved by removing those composites with a length of less than 0.25m and adjusting those with a length of greater than 0.25 m to 0.5 m to ensure all samples were given equal weight.

Adjusting the length of the footwall composites resulted in their location in 3D space being recalculated.    The maximum shift was 0.25 m in any direction.    This affected 84 samples.

The composited data was then separated into each of the three ore domains, Main Vein, Hanging Wall Split and Footwall Splay, and run through a block modeling function using the following parameters.

| Classification | Long axis | Azimuth | Plunge | Intermediate axis | Dip | Short axis |
| --- | --- | --- | --- | --- | --- | --- |
| Measured | 20 | 10 | 30* | 15 | 85* | 2 |
| Indicated | 40 | 10 | 30 | 30 | 85 | 4 |
| Inferred | 80 | 10 | 30 | 60 | 85 | 8 |

In the modeling system used a +ve Dip indicated an anti-clockwise rotation, in this case a dip to the West.

An IDP (inverse distance power) function was used for the weighting.    With the power set to 2.    The block size was as shown below.

| Direction | Size |
| --- | --- |
| East | 1 |
| North | 5 |
| RL (Elev) | 5 |

The same modeling parameters were applied to the Main Vein, Hanging Wall Split and Footwall Splay data sets.

The modeling process produced 9 files (3 each for the Main Vein, Hanging Wall Split and Footwall Splay) corresponding to Measured, Indicated and Inferred Categories of ore.

The modeling constraints referred to above were then applied to the block models using a sub blocking function to define which blocks in the model were inside each of the domains.

The models were then merges to give a full model for all three domains.    Because of the close proximity’s of the Main Vein and Hanging Wall Split and the Main Vein and Footwall Splay there was a possibility that the models contained some partial blocks that could be coalesced.    This was checked and 50 blocks were found in this category.    These were merged into the Main Vein and removed from the Hanging Wall Split and Footwall Splay models respectively.    When these blocks were merged the grades of the merged blocks were weighted using the portion of the block in each model to produce an adjusted grade.

The grades and tonnages in the two models were then reported as The NVG Project Geological Resource.    Only the figures dated February 11, 1998 should be used.

In Situ Mineable Reserve

Once the Geological Resource was calculated the In Situ Mineable Reserves were calculated.    To do this the stoping diagram attached was digitized and applied to the data in the Main Vein Model.    The blocks within the stoping outline were then reported under the categories Possible, Probable and Proven for all three ore domains.

Both the Hanging Wall Split and the Footwall Splay were classified as being single mining units and for this reason the Total In Situ Mineable Reserve in the Hanging Wall Split and the Footwall Splay is the same as the Total Geological Resource for these zones.

The mining outline used is that same as that used in the Feasibility study.    It should be noted that there is the potential for minor enlargement of this shape in the light of the Geological Resource referred to above.

The categories Possible Probable and Proven in the In Situ Mineable Reserve correspond to the categories Inferred, Indicated and Measured in the Geological Resource. (With the proviso that they are inside a mining block).    These are Australian AusIMM categories and I believe that ore in the Proven Category would also be classified as Probable in North America as it has not been tested by Underground Development.

The In Situ Mineable Reserve has been reported as The NVG Project In-situ Mineable Reserve.    Only the figures dated 11 February 1998 should be used.

The Diluted Mineable Reserve

The Diluted Reserves take the material in the mining shape and apply dilution and recovery factors.

The Factors used depend on the mining method and are set out below :

| Method | Dilution | Recovery |
| --- | --- | --- |
| Development | Variable | 100 |
| Avoca | 17 | 90 |
| Bench & Fill | 9 | 95 |
| Open Stoping | 7 | 95 |
| Pillar Recovery under fill | 25 | 85* |
| Shrink Stope | 15 | 100 |
| Rib Pillars | 30 | 70 |
| Hanging Wall Split | 15 | 100 |

The Development has a variable dilution, as it is dependent on the absolute width of the development.    The dilution during development coming from sidewalls of the drives.

A more detailed report on the reserve and the methodology is being prepared.