SIMATIC

STEP 7- Micro/WIN V4.0 SP6

Copyright (c) Siemens Energy & Automation Inc., 2007

Contents

Notes on Installation

1	Operating System and Hardware Requirements

2	Operating Environment

3	Installation

4	Installing the Upgrade License

Notes on Usage

# 5	What's New

# 6	Known Problems and Remedies

6.1	Problems

6.2	WIPEOUT.EXE

6.3	National Languages

6.4	General Communications

6.4.1	Multi-Master Communications

6.5 	Project Compatibility

7	Using the Force Function

Notes on Installation

1	Operating System and Hardware Requirements

- Microsoft Windows 2000 (Service Pack 3 or later), XP, or Vista.

- A personal computer (PC) capable of running one of the above operating systems, refer to Microsoft specifications for more information.

- At least 350 Mbytes of free hard disk space

- Use the small font setting and a minimum screen resolution of 1024x768 pixels

- Any mouse supported by Microsoft Windows

- For communicating with the S7-200 you will need one of the following:

–	PC/PPI Cable connected to your USB port

–	PC/PPI Cable connected to your serial communications port (PC COM1 or COM2)

–	Communications processor (CP) card and multipoint interface (MPI) cable

–	EM241 modem expansion module

–	CP243-1 or CP243-1 IT Ethernet expansion module

- Note: Communication card CP5613 and CP5614 are not supported.

- Note: Installation Problems with SIMATIC NET
If you have SIMATIC NET V6.2 or earlier installed on you computer you may receive the following error during installation.  
"Fatal error in SIMATIC NET Component sntieno.dll"
"Installation of protocol siem_isotrans failed with error code hr=0x80070430"

If you experience this error follow these steps:
1. Finish the installation and reboot your PC.  Rebooting is required.
2. Use Add/Remove Programs to uninstall the software and reboot your PC.  Rebooting is required.
3. Install the software again, this time you will not receive the error.

- Intelligent modules can be placed in any valid module position (if the CPU firmware is version 1.21 or greater). For systems containing earlier CPU firmware that supports intelligent modules, refer to Internet update 9261022. This update for earlier CPU firmware basically requires all intelligent modules to be located as a group adjacent to the CPU (for example: no discrete or analog module is allowed between the CPU and the group of intelligent modules).

Intelligent modules currently available:

–    EM277 PROFIBUS-DP

–    EM241 Modem

–    EM253 Position

–    CP243-1 Ethernet

–    CP243-1 IT Internet

2	Operating Environment

To be able to work with STEP 7-Micro/WIN software under Windows 2000, XP, or Vista, you must have logged on with at least power user privileges. You must have administrator rights for the installation of the STEP 7-Micro/WIN software and also for setting up modules using the Set PG/PC Interface application.

It is recommended that you disable hibernation mode when using STEP 7-Micro/WIN. Entering hibernation mode while online connections are open or while projects are being accessed by STEP 7-Micro/WIN through a network can cause the following errors:

- Unintentional termination of online connections

- Data loss in any open projects that were opened through a PC network

To avoid these errors, set the energy saving options for the operating system in the Windows Control Panel to manual trigger. This ensures that hibernation is not automatically triggered by a timer. Do not manually trigger hibernation mode during a communication task, as described above.

STEP 7-Micro/WIN Version 4.0 has been tested under the following operating system versions:

- Windows 2000 with SP3

- Windows XP

- Windows Vista

Only STEP 7-Micro/WIN V4.x releases can accommodate a simultaneous connection with S7-200 PC Access v1.x.  For instance you cannot run communications in 
STEP 7-Micro/WIN V3.x and S7-200 PC Access v1.x at the same time.

Note: If you experience unstable operation or unexplained program lockups, please examine the BIOS settings of your computer and make sure hyper-threading is disabled.  To resolve other situations please reboot your PC before running the application again.

3	Installation

1.  Close all applications including the MS-Office Toolbar.

2.  Insert the STEP 7-Micro/WIN CD into the CD-Rom drive. The Setup program will run automatically if you have not disabled the auto start option. You can manually start the Setup program by double clicking the file "Setup.exe" located on the CD.

3.  Follow the instructions displayed to complete the installation.

Note: 	To install STEP 7-Micro/WIN on a Windows 2000, XP, or Vista operating system, you must log in with administrator rights.

Note: 	When you install STEP 7-Micro/WIN, the installer program asks you to specify a target directory.  It is recommended that you remove previous versions of 
STEP 7-Micro/WIN prior to installing the new version.  Use "Add/Remove Programs" from the Control Panel to uninstall the previous version.

4	Installing the Upgrade License

If a previous version of STEP 7-Micro/WIN is already installed on the computer, simply run the upgrade installation from the Upgrade CD.  If you are installing to a new computer with a new operating system, first you should install the full Individual License, and then run the upgrade installation from the Upgrade CD.  If you experience problems installing the original Individual License on the new computer, please follow these steps:

1. Abort from the failed installation.

2. On the STEP 7-Micro/WIN V4.x Upgrade CD, locate the file named UpgradePatch.exe.

3. Run the file Upgrade_Patch.exe by double-clicking on the file.

4. Restart the upgrade installation by inserting the Upgrade License CD.

Notes on Usage

5	What's New

To find out what's new in V4.0, run the STEP 7-Micro/WIN application and double-click on the "What's New" link at the top of the project tree.

6	Known Problems and Remedies

6.1	Problems

1. 	Windows 2000 auto-detection of a serial port mouse.

A computer with the Windows 2000 operating system attempts to auto-detect a serial port mouse.  If your computer is attached to a S7-200 multi-master network when the operating system starts, then Windows 2000 incorrectly reports the S7-200 network traffic as a serial port mouse. In this state, random mouse movement and right clicks will appear on the Windows 2000 desktop. You must disconnect the S7-200 network to stop the incorrect data transmission. This Windows 2000 problem is known to exist on Windows 2000 versions SP1 and SP2 and is discussed in Microsoft Knowledge Base article Q283063.

If you experience this problem, please try the following solutions:

- Disconnect the S7-200 serial port connector before powering up or rebooting Windows 2000.  After the boot process is complete and you are logged in, reconnect the computer to the S7-200 network.

- If you do not use a serial mouse, you can modify your boot.ini file to include the /NoSerialMouse switch option. Use the Notepad text editor to modify [operating system] lines in the boot.ini file. The boot.ini file is located in the root directory of the boot drive (typically C:\).

For example, if the boot.ini lists:

[operating system]

multi (0) disk (0) rdisk (0) partition (1) \WINNT="Microsoft Windows 2000 Professional" /fastdetect /noguiboot

Then add the /NoSerialMouse switch, as shown below:

multi (0) disk (0) rdisk (0) partition (1) \WINNT="Microsoft Windows 2000 Professional" /fastdetect /noguiboot /NoSerialMouse

2. 	CP 5511 Card on Notebook PC

Internally, the CP 5511 card is ISA based, and the new technology of Plug & Play does not work properly. STEP 7-Micro/WIN and all SIMATIC products install an .inf file that contains the most common configurations for most PCs.  However, not all PCs are designed the same, and you may need to obtain a different .inf file from the S7-200 hotline. If you experience problems getting the CP 5511 card to install or function properly, please call the Siemens S7-200 hotline.

3. 	Installing CP Cards

You should always install STEP 7-Micro/WIN before inserting and installing any CP card. STEP 7-Micro/WIN contains I/O driver software that is used by CP cards.

4.	Managing Applications Access to the Serial Port

If you use the STEP 7-Micro/WIN Communications Setup or the PC/PG Interface under the PC's Control Panel to set up the PC COM PORT parameters, certain conditions will cause the COM Port selection field to be shown with an asterisk beside the COM Port number. This asterisk indicates that the user's PC setup has already assigned another application to use the same COM Port. For example, if the Palm Pilot software is installed on the PC and you try to install STEP 7-Micro/WIN, this asterisk will appear because Palm Pilot software typically loads each time the PC is booted up. You must unload (not uninstall) the other application to free up the COM Port so that 
STEP 7-Micro/WIN can gain access to the port.

5. 	HP DeskJet Color Printers

If you are experiencing problems with an HP DeskJet color printer, or you are experiencing problems with other printers, try downloading the newest available drivers from the printer manufacturer. Otherwise, you can change the printer driver to the Windows universal driver for your printer. For example (in the case of the DeskJet), the universal driver is the 550c driver.

6. 	TD 200 Communication

A problem can occur during downloading or uploading a program using a CPU 215 or CPU 216 when an older TD 200 (pre-v1.2) is polling the CPU. The current time-out error on the TD 200 is no longer appropriate for new CPU models. To work around this error, temporarily disconnect the TD 200 from the network, download or upload the program, and then reconnect the TD 200 to the network.

7. 	TC-35 Cell Phone Modems

Certain Dell laptops used with a Siemens TC-35 modem, will not properly communicate. The problem is seen with some Dell laptops that drive the transmit line high when the serial port is not in use. Because of this, the Siemens TC-35 (and possibly other industry standard modems) resets causing initialization sequencing problems followed by communication errors.

8. 	TC-35 Cell Phone Modem Remote Programming

Remote modem programming of a Siemens TC-35 modem uses an &W AT-command. Early firmware versions of this modem do not support this command. Programming of this modem will fail, if this command is not supported.

9. 	Network Printers

If a network printer is assigned as the default printer and the PC network connection is disconnected, there may be an error when starting STEP 7-Micro/WIN. Reconnect the PC to the network or assign a local printer as the default printer and restart 
STEP 7-Micro/WIN.

10. 	S7-200 Documentation CD

If you are experiencing problems with the "Programming Tips" section of the S7-200 Documentation CD, you may need to install or update the Java Runtime Environment. To download a free plug-in for this feature, go to http://www.java.com and select the "Free Downloads" link. Follow the instructions for installing the Java Runtime Environment, the re-start the S7-200 Documentation browser.

11. 	A box instruction that is connected directly to the left vertical rail does not always execute. In the LAD network shown below, the DEC_B box will not execute every program scan. The DEC_B box in network 2 will execute only when the result of network 1 evaluates as TRUE (Q0.0=1) and will not execute if network 1 evaluates to FALSE (Q0.0).

12. 	A new Microsoft Vista operating system installation cannot display the STEP 7-Micro/WIN help (*.hlp) files because the required winhelp32.exe help viewer is not installed. To view Micro/Win Help topics, you must use an internet connection and download the new winhelp32.exe file directly from the Microsoft download center.

Go to http://support.microsoft.com/kb/917607 and follow the instructions. You must download the Genuine Windows Validation component to get access to the download page where you can download the Windows 6.0-KB917607-x64.msu file (for a 64-bit PC) or Windows 6.0-KB917607-x86.msu (for a 32-bit PC).

- Special Characteristics of the Microsoft Windows Vista Operating System:

Note that the software in MS Windows Vista is no longer under "Start menu > SIMATIC"; it is now located under "Start menu > Program Files > Siemens Automation > SIMATIC". This was made necessary by the current architecture of MS Windows Vista.

6.2	WIPEOUT.EXE

The WIPEOUT utility is a Command Prompt (Console) application, which restores the PLC to factory defaults.  This program is useful if you have a PLC at an unknown address or baud rate and you cannot communicate to the PLC using STEP 7-Micro/WIN.  Before using WIPEOUT, try the "Search all baud rates" feature on the Communications dialog.

The WIPEOUT utility requires an RS-232 PC/PPI Cable.  If using the RS-232 PC/PPI Multi-master Cable, switch 5 must be off.  You can not use WIPEOUT with a USB cable.

The WIPEOUT utility will delete the user program (OB1), the data block (DB1), and all configuration information (SDB).  The PLC will then be set at address 2 at 9600 baud. The WIPEOUT utility is not installed with STEP 7-Micro/WIN. A copy of the utility is included on the STEP 7-Micro/WIN installation CD.  The utility may be run from the CD or copied and run from the hard drive. To run the WIPEOUT utility, open a Command Prompt and type "WIPEOUT," and follow the directions.

6.3	National Languages

STEP 7-Micro/WIN uses several Microsoft common dialog boxes. The text in these dialogs appear in the national language of the operating system, regardless of the language you have chosen for STEP 7-Micro/WIN.

6.4	General Communications

Refer to the S7-200 Programmable Controller System Manual located on the Documentation CD for a list of communication configurations.

6.4.1	Multi-Master Communications

Note:  When using Multi-Master communications on networks with several masters (including TDs or OPs), the hardware takes several seconds to initialize onto the PLC network. If communication is attempted before this initialization is complete, an error may occur. Retrying the last request should correct it.

Setting STEP 7-Micro/WIN Network Addresses:

As a general rule, use the lower available addresses for masters and the higher available addresses for slaves. It is preferable (but not required) to use consecutive addresses for the masters on a network. For example:

## Network Components	Possible Addresses

STEP 7-Micro/WIN	0

TD 200	1

OP15	2

Several S7-200 slaves	3, 4, and 5 You could leave these numbers free for additional masters and start the numbering of the S7-200 slaves at 16.

TD 200-MPI:

A problem can occur during downloading or uploading a program with a CPU 215 or CPU 216 when an older TD 200 (pre-v1.2) is polling the CPU. The current time-out error on the TD 200 is no longer appropriate for these CPU models. To workaround this error, temporarily disconnect the TD 200 from the network, download or upload the program, and then reconnect the TD 200 to the network.

Master Mode Communications Error:

STEP 7-Micro/WIN might report erroneous communications time-out errors when the PLC is in Master mode. For example, using STEP 7-Micro/WIN to put the CPU in STOP mode might return this time-out error even though the CPU did go to STOP mode.

6.5	Project Compatibility

1.  In V4.x when uploading a data block from a CPU 21x PLC, all data block formatting information and comments will be deleted.

2.  Projects prior to V3.0 which have double quotes embedded in strings will not compile.  The projects must be modified as described under the Illegal Syntax rules located in the help system.

3.  Local memory usage is no longer supported on String operands.

4.  In rare cases with extremely large programs, there is a new option of downloading the program without downloading wizard configurations.  The program will download and execute properly, but subsequent uploads will not allow re-running wizards to edit the uploaded program.  In previous releases the download would have been blocked.

### 7	Using the Force Function

Warning!

When you force a value in your program, the operand is reset to that value with each scan of your program, regardless of input/output conditions or other program logic that would ordinarily have an effect on the operand value. Forcing can cause your process to operate unpredictably, and unpredictable process operation can cause death or serious injury to personnel, and/ or damage to equipment. The Force function is an aid for debugging your program; never force a value to compensate for problems with your process equipment. Only experienced personnel should use the Force function. Be sure that all personnel who have authority to maintain or debug your process are aware when program values have been forced.

Consider the following when using the Force function:

- Any forced values will remain forced even when the PLC is in STOP mode.

- The Force function overrides an output that has been configured to go to a specified value when the PLC transitions to STOP mode.

- If the PLC is power cycled, any forced values will be reapplied when power is restored to the PLC.

For more information regarding the Force function, consult the S7-200 Programmable Controller System Manual.